TECHNICAL EVALUATION report

FOR PVC pipe manufacturer

SAMSON INTERNATIONAL PLC

October 2018

PVC Pipes & Fittings Manufacturers

(01)	Introduction

SBDRC studied the Technical suitability of PVC pipe & fittings manufacturers in Sri Lanka in order to conform the suitability of their products to be used for water supply systems.

(02)	NWSDB Requirements for upvc pipes & fittings

Manufacturer shall have the following certificates issued by SLSI or accredited agency who is a member of IAF.

a.	SLS 147:2013, Product Conformity Certificate for uPVC pipes.

b.	ISO 9001:2015, Quality Management System Certificate for the pipe manufacturing factory.

c.	SLS 659:2015 or ISO 1452-Part 3 Product Conformity Certificate for Mechanical welded fittings (Rubber ring joints)/ solvent welded fittings.

d.	ISO 9001:2015, Quality Management System Certificate for the fittings manufacturing factory.

e.	SLS 935:1991 for solvent cement.

f.	ISO 9001: 2015, Quality Management System Certificate for the solvent cement manufacturing factory.

g. 	If DI fittings are provided instead of uPVC fittings.

(i)EN 12842 / EN 545/ ISO 2531, Product Conformity Certificate for DI fittings.

(ii)ISO 9001: 2015, Quality Management System Certificate for DI fittings manufacturing factory.

h.	BSEN 681-1 / ISO 4633 /SLS ISO 4633, Product Conformity Certificate for joint rings.

- ISO 9001: 2015 Quality Management System Certificate for joint rings manufacturing factory.

j          Guarantee from pipe manufacturer to ensure a leak proof pipe line.

k	Guarantee to supply Rubber Ring to uPVC pipe manufacturer.

(03)	Present SITUATION

At present NWSDB has short listed seven pipe Manufacturers for uPVC pipes and fittings. The approved contractors with their details are as follows.

- St. Anthony’s Industries Group (Pvt.) Ltd has short listed to supply uPVC pipes, fittings and solvent cement with joint rings from Samson International PLC.

- Central Industries PLC has short listed to supply uPVC pipes, fittings and solvent cement with joint rings from Samson International PLC.

- S-lon Lanka (Pvt) Ltd has short listed to supply uPVC pipes, fittings and solvent cement with joint rings from Samson International PLC.

- Kamal PVC Industries (Pvt.) Ltd has short listed to supply uPVC pipes, fittings and solvent cement with joint rings from Samson International PLC.

- Sierra Industries (Pvt.) Ltd has short listed to supply uPVC pipes, fittings with solvent cement from Central Industries PLC and joint rings Samson International PLC.

- Duro Pipe Industrial (Pvt) Ltd has short listed to supply uPVC pipes and solvent cement with uPVCfittings from Sierra Industries (Pvt.) Ltd and joint rings from Samson International PLC.

- Samson International PLC has short listed to supply uPVC pipes, fittings, joint rings and solvent cement.

(04)	TECHNICAL REVIEW Of Manufacturer

Samson International PLC

M/s. Samson International PLC has provided SLS Certificates to the NWS&DB to enhance the scope of their Pre-Qualification in uPVC Pipes and fittings. The manufacturer has already been included in the pre-qualification list for supply of uPVC pipes, fittings, rubber rings and solvent cement of the following scope.

uPVC pipes :

| Type | Nominal Outside Diameter, (mm) |
| --- | --- |
| PNT 14 | 20, 110 |
| PNT 11 | 25-90 |
| PNT 9 | 110 |
| PNT 7 | 32-90 |

uPVC Fittings:

| Product | Type | Size (mm) |
| --- | --- | --- |
| 900Elbow | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63 |
| Equal Tee | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63 |
| Valve Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63 |
| Faucet Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63 |
| Plain Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63 |

Now the manufacturer has requested to be included in the pre-qualification list for pipes upto 225mm dia and fittings from Prince Plastics Industries (pvt) Ltd for larger diameters.

The manufacturer has submitted the following certificates and required details for the Pre-Qualification and the position of the certificates produced by the manufacturer is given below.

This manufacturer posses,

  - SLS 147:2013, Product Conformity Certificates (PCC) for uPVC pipes of following sizes;

| Type | Nominal Outside Diameter, (mm) |
| --- | --- |
| PNT 14 | 20, 110 |
| PNT 11 | 25-160 |
| PNT 9 | 140,225 |
| PNT 7 | 32-225 |

This certificate is valid upto 06th April 2019. The certificate was verified as authentic by SLS and attached as Annex I.

  - SLS 659:2015, Product Conformity Certificates (PCC) for uPVC pipe joints and fittings for M/s Prince Plastic Industries (Pvt) Ltd. of following sizes;

| Product | Type | Size (mm) |
| --- | --- | --- |
| End Cap | PNT 14, | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| 900Elbow | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| Equal Tee | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| Equal Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| Valve Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| Faucet Tee | PNT 14 | 20 |
| Faucet Elbow | PNT 14 | 20 |
| Faucet Socket | PNT 14 | 20 |
|  | PNT 11 | 25,32,40,50,63,75,90,110 |
| Tapping Saddles | 50x20, 50x25, 50x32 mm 63x20, 63x25, 63x32, 63x40, 63x50 75x20, 75x25, 75x32, 75x40, 75x50 90x20, 90x25, 90x32, 90x40, 90x50 110x20, 110x25, 110x32, 110x40, 110x50, 110x63 160x20, 160x25, 160x32, 160x40, 160x50, 160x63 225x20, 225x25, 225x32, 225x40, 225x50, 225x63 280x20, 280x25, 280x32, 280x40, 280x50, 280x63 |  |

This Certificate is Valid upto 31st of December 2018. The certificate was verified as authentic by SLS and attached as Annex II.

  - SLS 935:1991 PCC for solvent cement.

This Certificate is Valid upto 17th November 2018. The certificate was verified as authentic by SLS and attached as Annex III.

  - ISO 9001:2015 Quality Management System Certificate for pipes, Solvent cement manufacturing factory.

This Certificate is Valid up to 11th of November 2018. The certificate was verified as authentic by DNV.GL-Business Assurance and attached as Annex IV.

  - ISO 9001:2015 Quality Management System Certificate for fittings manufacturing factory.

This Certificate is Valid up to 07th May 2021. The certificate was verified as authentic by SLS and attached as Annex V.

  - Instead of EN 681-1:1996 they all have produced a Test Certificate for joint rings from ITI and attached as Annex VI.

  - Guarantee from pipe manufacturer to ensure a leak proof pipe line and attached as Annex VII.

  - Guarantee to supply Fittings to pipe manufacturer and attached as Annex VIII.

(05)	Recommendation

having studied on the documents and certificates submitted from M/s Samson International PLC, This manufacturer is recommended to be short listed manufacturer to supply of PVC pipes up to 225mm dia, fittings up to 63mm diameter from M/s Samson International PLC, and from 75mm to 110mm diameter from M/s Prince Plastics Industries (pvt) Ltd .The scope as detailed in Table 1 given below.

Standard Bidding Review Committee :

- Eng. R.H. Ruvinis 			    Addl.GM (WSP) -	Chairman	………………

- Eng. N.H.R. Kulanatha		    Addl. GM (S/E) -  Member	 	……………...

- Eng. U. Ratnapala			    DGM (P&D) – Member		………………

- Eng. R.A.A. Ranawaka 		    PD (SHIFT Project) – Member	………………

- Eng. D.N. De S. Gunathilake		    DGM (Sewerage) – Member	………………

- Eng. S.S.S. Vipulanandan		    DGM (Head Office) – Member	……………...

- Eng. T.M.W. Sunil Bandara		    DGM (M&E-Services) – Member	………………

- Mr. A.G.S. Kumara 			    DGM (Costing) – Member		……………...

- Eng. J.A.K.K.C. Senevirathne	    AGM (T&C) – Member		………………

- Eng. T.W.M.L.P.Wijesundara	    AGM(Documentation) – Member	………………

- Eng. (Mrs.) D.S.P. R.D. Premachandra  CE (P&D-Doc)- Secretary		………………

Table 01

Details of Pre-Qualification Scope for PVC Pipes, Fittings and Rubber Rings

M/s. Samson International PLC (Pvt.) Ltd.,

| Manufacturers Name | Factory Address | Scope for pipes & fittings |  | Factory address for joint rings | Factory address for Solvent Cement |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure class | Size (mm) |  |  |
| Pipes | Pipes | PNT 14 | 20,110 | M/s. Samson International PLC, Gilcroft Estate, Kiribathawila, Baddegama | M/s. Samson International PLC, Gilcroft Estate, Kiribathawila, Baddegama |
| M/s. Samson International PLC, Akuressa Road, Bogahagoda, Angulugaha, Galle | M/s. Samson International PLC, Gilcroft Estate, Kiribathawila, Baddegama |  |  |  |  |
|  |  | PNT 11 | 25-160 |  |  |
|  |  | PNT 9 | 140,225 |  |  |
|  |  | PNT 7 | 32-225 |  |  |
| Fitttings | Fitttings |  |  |  |  |
| M/s. Samson International PLC, Akuressa Road, Bogahagoda, Angulugaha, Galle M/s Prince Plastic Industries (Pvt) Ltd, Industries (Pvt) Ltd, No 11,Messenger Street, Colombo 12 | M/s. Samson International PLC, Gilcroft Estate, Kiribathawila, Baddegama M/s Prince Plastic Industries (Pvt) Ltd, No 226,Negombo Road, Peliyagoda | PNT 14 | 20 |  |  |
|  |  | PNT 11 | 25-63 |  |  |
|  |  | PNT 11 | 75-110 |  |  |

Note: Higher diameter fittings can be imported if necessary with the SLS approval at the port.