SAMPLE BOQ

Bill No. 08

225,000 Lts. Capacity Water Tower

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS D – DIMOLISHING AND SITE CLEARING |  |  |  |  |
|  |  | General clearance |  |  |  |  |
|  | D 100 | Clearing site including grubbing up all small trees having girth not exceeding 500 mm and removal of debris filling voids with selected earth. | ha |  |  |  |
|  |  | CLASS E – EARTHWORKS Rate to include for dewatering and earthwork support if found necessary |  |  |  |  |
|  |  | Excavation for foundations |  |  |  |  |
|  | E 324 | Excavation in pit for main structure foundation, material other than rock or artificial hard material with maximum depth. 1.0 – 2.0 m. from ground level. | m3 |  |  |  |
|  | E 334 | Excavation in rock for main structure and, with maximum depth 1.0 – 2.0 m. | m3 |  |  |  |
|  |  | Excavation Ancillaries |  |  |  |  |
|  | E 522 | Preparation of excavated surfaces for concrete base, material other than top soil, rock or artificial hard material. | m2 |  |  |  |
|  | E 532 | Disposal of excavated material, material other than rock or artificial hard material away from site as directed. | m3 |  |  |  |
|  | E 533 | Disposal of rock away from site as directed. | m3 |  |  |  |
|  |  | Filling |  |  |  |  |
|  | E 614 | Filling to structures up to 900 mm depth in 150 mm thick layers with selected excavated material other than top soil or rock. | m3 |  |  |  |
|  | E 614 | Filling under floor up to 900 mm thick in 150 mm thick layers with selected excavated material other than top soil or rock. | m3 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Filling Ancillaries |  |  |  |  |
|  | E 722 | Preparation of filled surfaces for concrete base, material other than top soil, rock or artificial hard material. | m2 |  |  |  |
|  |  | CLASS F – IN SITU CONCRETE Reinforcement and formwork measured separately unless otherwise stated |  |  |  |  |
|  |  | Provision of Concrete |  |  |  |  |
|  | F 228 | Designed mix Grade C 10 Ordinary Portland Cement 40 mm aggregate. | m3 |  |  |  |
|  | F 247 | Designed mix Grade C 20 Ordinary Portland Cement 20 mm aggregate. | m3 |  |  |  |
|  | F 257 | Designed mix Grade C 25 Ordinary Portland Cement 20 mm aggregate. | m3 |  |  |  |
|  |  | Placing of Concrete (Grade C 10) Mass Concrete |  |  |  |  |
|  | F 511 | In screed for foundation base with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 543 | In side walls at columns with thickness exceeding 300 mm but not exceeding 500 mm. | m3 |  |  |  |
|  | F 521 | In floor of tower with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 522 | In step at entrance with thickness exceeding 150 mm but not exceeding 300 mm. | m3 |  |  |  |
|  |  | Placing of Concrete (Grade C 20) Reinforced Concrete |  |  |  |  |
|  | F 624 | In base and footing of foundation with maximum thickness exceeding 500mm. | m3 |  |  |  |
|  | F 653 | In columns up to bracing No. 1 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS F – IN SITU CONCRETE (CONDT.) Placing of Concrete (Grade C 20) Reinforced Concrete |  |  |  |  |
|  | F 653 | In columns from bracing No. 1 to bracing No. 2 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 653 | In columns from bracing No. 2 to bracing No. 3 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 653 | In columns from bracing No. 3 to bracing No. 4 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 653 | In columns from bracing No. 4 to bottom of beam with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 663 | In bracing No. 1 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 663 | In bracing No. 2 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 663 | In bracing No. 3 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 663 | In bracing No. 4 with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 666 | In beams under tank. | m3 |  |  |  |
|  | F 663 | In circular outer ring beam at tank base level with cross sectional area exceeding 0.1 m2 but not exceeding 0.25 m2. | m3 |  |  |  |
|  | F 662 | In circular inner ring beam at tank base level with cross sectional area exceeding 0.03m2 but not exceeding 0.1m2. | m3 |  |  |  |
|  | F 662 | In lintels with cross sectional area exceeding 0.03m2 but not exceeding 0.1m2. | m3 |  |  |  |
|  | F 631 | In sun shed with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 662 | In beam under platform No. 3 with cross sectional area exceeding 0.03m2 but not exceeding 0.1m2. | m3 |  |  |  |
|  | F 631 | In platform No. 1 with thickness not exceeding 150 mm. | m3 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS F – IN SITU CONCRETE (CONDT.) Placing of Concrete (Grade C 20) Reinforced Concrete |  |  |  |  |
|  | F 631 | In platform No. 2 with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 631 | In platform No. 3 with thickness not exceeding 150 mm. | m3 |  |  |  |
|  |  | Placing of Concrete (Grade C 25) Reinforced Concrete |  |  |  |  |
|  | F 621 | In tank base with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 641 | In circular inner shaft of tank including 125 x 125 mm chamfer with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 642 | In circular outer walls of tank including 125 x 125 mm chamfer with thickness exceeding 150 mm. but not exceeding 300 mm. | m3 |  |  |  |
|  | F 621 | In roof slab of tank with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 641 | In circular walls of lantern with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 621 | In roof slab of lantern with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 641 | In circular curb wall on edge of roof slab with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 631 | In platform No. 4 with thickness not exceeding 150 mm. | m3 |  |  |  |
|  | F 69 | In 375x375x150 mm pipe bracket complete as shown in detail on drawing. Rate to include for formwork. | m3 |  |  |  |
|  |  | CLASS G – CONCRETE ANCILLARIES |  |  |  |  |
|  |  | Form Work Form work in approved timber or any other suitable material. Rate to include for splayed edges, battens, strutting, bolting, wedging, easing, striking and removal to the approval. |  |  |  |  |
|  |  | Rough finish |  |  |  |  |
|  | G 153 | For circular sides of foundation base having 11.1 m dia. with width 0.2 – 0.4 m. | m2 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Form Work Rough finish (Condt.) |  |  |  |  |
|  | G 153 | For circular internal sides of column footing having 5.4 m dia. with width 0.2 – 0.4 m. | m2 |  |  |  |
|  | G 141 | For 100 mm high column kickers. | m |  |  |  |
|  |  | Fair Finish Suitable material shall be used for wrought formwork, so that surface shall have very neat and smooth finish after removal to the approval. |  |  |  |  |
|  | G 244 | For sides of side walls at ground level with width 0.4 – 1.22 m | m2 |  |  |  |
|  | G 243 | For sides of side walls at ground level with width 0.2 – 0.4m | m2 |  |  |  |
|  | G 223 | For sloping top face of side walls at ground level with width 0.2 – 0.4m | m2 |  |  |  |
|  | G 282 | For sides of 600 x 375 mm columns form top of base to top of bracing No. 1 (8 nr. Columns) | m |  |  |  |
|  | G 282 | For sides of 600 x 375 mm columns form top of bracing No. 1 to top of bracing No. 2 (8 nr. Columns) | m |  |  |  |
|  | G 282 | For sides of 600 x 375 mm columns form top of bracing No. 2 to top of bracing No. 3 (8 nr. Columns) | m |  |  |  |
|  | G 282 | For sides of 600 x 375 mm columns form top of bracing No 3 to top of bracing No. 4 | m |  |  |  |
|  | G 282 | For sides of 600 x 375 mm columns form top of bracing No. 3 to bottom of beam under tank base (8 nr. Columns) | m |  |  |  |
|  | G 281 | For sides and soffits of 300 x 525 mm bracing No. 1. | m |  |  |  |
|  | G 281 | For sides and soffits of 300 x 525 mm bracing No. 2. | m |  |  |  |
|  | G 281 | For sides and soffits of 300 x 525 mm bracing No. 3. | m |  |  |  |
|  | G 281 | For sides and soffits of 300 x 525 mm bracing No. 4. | m |  |  |  |
|  | G 244 | For sides of beams under tank with width 0.4 - 1.22 m | m2 |  |  |  |
|  | G 243 | For sides of beams under tank with width 0.2 – 0.4 m | m2 |  |  |  |
|  | G 223 | For sloping sides of beams under tank with width 0.2 – 0.4m | m2 |  |  |  |
|  | G 281 | For sides and soffits of 450 x 300 mm circular ring beam under tank base. | m |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CONCRETE ANCILLARIES (CONDT.) Form Work Fair finish |  |  |  |  |
|  | G 281 | For sides and soffits of 225 x 225 mm circular ring beam under tank base. | m |  |  |  |
|  | G 215 | For soffit of tank base with width exceeding 1.22 m | m2 |  |  |  |
|  | G 251 | For 100 mm high circular wall kickers having 8750mm dia. | m |  |  |  |
|  | G 251 | For 100 mm high circular wall kickers having 1550mm dia. | m |  |  |  |
|  | G 252 | For conical sides of chamfer having 8075 mm mean dia. with width 0.1 – 0.2 m | m |  |  |  |
|  | G 252 | For conical sides of chamfer having 1925 mm mean dia. with width 0.1 – 0.2 m | m |  |  |  |
|  | G 252 | For circular sides of tank base having 8750 mm dia. with width 0.1 – 0.2 m | m |  |  |  |
|  | G 252 | For circular sides of tank base having 1550 mm dia. with width 0.1 – 0.2 m | m |  |  |  |
|  | G 255 | For circular sides of tank wall having 8200 mm dia. with width exceeding 1.22 m | m2 |  |  |  |
|  | G 254 | For circular sides of tank wall having 8750 mm dia. with width 0.4 - 1.22 m | m2 |  |  |  |
|  | G 255 | For circular sides of inner shaft wall having 1550 mm dia. with width exceeding 1.22 m | m2 |  |  |  |
|  | G 255 | For circular sides of inner shaft wall having 1800 mm dia. with width exceeding 1.22 m | m2 |  |  |  |
|  | G 255 | For circular sides of tank wall having 8550 mm dia. with width exceeding 1.22 m | m2 |  |  |  |
|  | G 211 | For soffit of projection of circular tank wall with width not exceeding 0.1 m | m |  |  |  |
|  | G 215 | For soffit of tank roof with width exceeding 1.22 m | m2 |  |  |  |
|  | G 215 | For soffit of lantern roof with width exceeding 1.22 m | m2 |  |  |  |
|  | G 252 | For circular sides of curb wall having 8550 mm dia. with width 0.1 – 0.2 m | m |  |  |  |
|  | G 251 | For circular sides of lantern roof having 1950 mm dia. with width not exceeding 0.1 m | m |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CONCRETE ANCILLARIES (CONDT.) Form Work Fair finish |  |  |  |  |
|  | G 281 | For sides and soffits of 225 x 150 mm lintel. | m |  |  |  |
|  | G 214 | For soffit of sunshade with width 0.4 - 1.22 m | m2 |  |  |  |
|  | G 241 | For edges of sunshade with width not exceeding 0.1 m | m |  |  |  |
|  | G 214 | For soffit of platform No. 1 & No.2 with width 0.4 - 1.22 m | m2 |  |  |  |
|  | G 281 | For sides and soffits of 225 x 375 mm beam under platform No. 3 | m |  |  |  |
|  | G 214 | For soffit of platform No. 3 with width 0.4 - 1.22 m | m2 |  |  |  |
|  | G 213 | For soffit of platform No. 4 with width 0.2 – 0.4 m | m2 |  |  |  |
|  | G 241 | For edges of platforms with width not exceeding 0.1 m | m |  |  |  |
|  | G 290 | For forming holes for 200 mm dia. DI pipe passing through 125 mm thick tank wall. | nr |  |  |  |
|  | G 290 | For forming holes for 200 mm dia. DI pipe passing through 150 mm thick tank base. | nr |  |  |  |
|  | G 290 | For forming holes for 250 mm dia. DI pipe passing through 150 mm thick tank base. | nr |  |  |  |
|  |  | Reinforcement Tor steel / Mild steel rod reinforcement cut to lengths, bent to shape, placed in position and tied with GI wire |  |  |  |  |
|  |  | Deformed high yield steel bars to BS 4449 |  |  |  |  |
|  | G 524 | 25 mm dia. | t |  |  |  |
|  | G 524 | 20 mm dia. | t |  |  |  |
|  | G 524 | 16 mm dia. | t |  |  |  |
|  | G 524 | 12 mm dia. | t |  |  |  |
|  | G 523 | 10 mm dia. | t |  |  |  |
|  |  | Plain round steel bars to BS 4449 |  |  |  |  |
|  | G 511 | 6.3 mm dia. | t |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS H – PRE-CAST CONCRETE (Rate to include for reinforcement and formwork as indicated on drawing and as directed) |  |  |  |  |
|  |  | Concrete Louvers |  |  |  |  |
|  | H 9 | Concrete louvered window of size 600 x 1500 mm with 20 Nr. 20 mm thick pre-cast concrete louver blades 115 mm wide and 600 mm long with 40 x 20 mm winged section for insertion in to groove provided on the vertical sides of pre-cast concrete frame complete as shown in details on drawing. | nr |  |  |  |
|  | H 601 | 400 X 400 X 300 mm pre-cast concrete grade 10 support blocks for duck foot bends. | nr |  |  |  |
|  |  | CLASS J – PIPE WORK – FITTINGS AND VALVES |  |  |  |  |
|  |  | Supplying, laying and jointing or fixing in position DI/CI/PVC pipes, specials, fittings and other materials as shown in detail on drawing and as directed. Rate to include for concrete block supports where required and as specified. |  |  |  |  |
|  |  | Inlet |  |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 3000 mm long. | nr |  |  |  |
|  | J 311 | 200 mm dia. DI double flanged duck foot bend. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI flanged and plain ended pipe 1000 mm long. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI flanged and plain ended pipe 4000 mm long. | nr |  |  |  |
|  | J 811 | 200 mm dia. Double flanged Sluice Valve with operating wheel. | nr |  |  |  |
|  | J 351 | Flanged adapter to suit 200 mm dia. DI pipe. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 4000 mm long. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 925 mm long. | nr |  |  |  |
|  | J 311 | 200 mm dia. DI double flanged 90 deg. bend. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI flanged and plain ended pipe 2100 mm long. | nr |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS J – PIPE WORK – FITTINGS AND VALVES (CONDT.) |  |  |  |  |
|  | J 381 | 200 mm dia. DI Double flanged pipe 1700 mm long. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 500 mm long with puddle flange 240 mm from one end. | nr |  |  |  |
|  | J 891 | 200 mm dia. ball float valve. | nr |  |  |  |
|  |  | Outlet |  |  |  |  |
|  | J 381 | 250 mm dia. DI flanged and plain ended pipe 650 mm long with puddle flange 150 mm from plain end. | nr |  |  |  |
|  | J 381 | 250 mm dia. DI double flanged pipe 4000 mm long. | nr |  |  |  |
|  | J 381 | 250 mm dia. DI flanged and plain ended pipe 2850 mm long. | nr |  |  |  |
|  | J 351 | Flanged adapter to suit 250 mm dia. DI pipe. | nr |  |  |  |
|  | J 811 | 250 mm dia. Double flanged Sluice Valve with wheel. | nr |  |  |  |
|  | J 381 | 250 mm dia. DI double flanged pipe 1000 mm long. | nr |  |  |  |
|  | J 311 | 250 mm dia. DI double flanged duck foot bend. | nr |  |  |  |
|  | J 381 | 250 mm dia. DI double flanged pipe 3000 mm long. | nr |  |  |  |
|  |  | # Overflow & Scour |  |  |  |  |
|  | J 371 | 200 mm dia. DI flanged bell mouth. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 4000 mm long. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 850 mm long with puddle flange 425 mm from one end. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI Double flanged pipe 1220 mm long. | nr |  |  |  |
|  | J 351 | Flanged adapter to suit 200 mm dia. DI pipe. | nr |  |  |  |
|  | J 311 | 200 mm dia. DI double flanged duck foot bend. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 650 mm long. | nr |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS J – PIPE WORK – FITTINGS AND VALVES (CONDT.) |  |  |  |  |
|  | J 381 | 200 mm dia. DI flanged and plain ended pipe 625 mm long with puddle flange 100mm from plain end. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI flanged and plain ended pipe 1150 mm long. | nr |  |  |  |
|  | J 811 | 200 mm dia. Double flanged Sluice Valve with wheel. | nr |  |  |  |
|  | J 321 | 200 x 200 mm dia. DI all flanged tee. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 2450 mm long. | nr |  |  |  |
|  | J 381 | 200 mm dia. DI double flanged pipe 3000 mm long. | nr |  |  |  |
|  |  | # Roof Drain |  |  |  |  |
|  | J 481 | 63 mm dia. PVC pipe piece 300 mm long. | nr |  |  |  |
|  | J 411 | 63 mm dia. PVC 90 deg. socketed bend. | nr |  |  |  |
|  |  | CLASS N: MISCELLANEOUS METALWORK Supplying and fixing in position as shown on drawing following items and as directed |  |  |  |  |
|  |  | Stainless steel Handrail |  |  |  |  |
|  | N 140 | 900 mm high stainless steel tubular circular hand railing on roof slab, assembled out of 32 mm dia. stainless steel pipes and fittings welded to 8 mm thick 150 x 150 mm stainless steel base plates and the base plate fixed to the concrete with 4 Nr. 12 mm dia. 100 mm stainless steel anchor bolt and nuts. | m |  |  |  |
|  | N 140 | 900 mm high stainless steel tubular straight hand railing on platforms, assembled out of 32 mm dia. stainless steel pipes and fittings welded to 8 mm thick 150 x 150 mm stainless steel base plates and the base plate fixed to the concrete with 4 Nr. 12 mm dia. 100 mm stainless steel anchor bolt and nuts. | m |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS N: MISCELLANEOUS METALWORK (CONDT.) |  |  |  |  |
|  |  | # Aluminium Ladders Aluminium ladder with 66.7 x 31.7 mm Aluminium stringers 375 mm apart and 24.9 mm dia. Aluminium non slip rungs at 300 mm centers and ladder fixed to concrete slab at top and bottom with 76.2 x 50.8 x 6.35 mm Aluminium angle cleat and 12 mm dia. stainless steel bolts and nuts. Ladder complete with Aluminium safety hoops as shown in detail on drawing. |  |  |  |  |
|  | N 130 | Ladder No. 1 | m |  |  |  |
|  | N 130 | Ladder No. 2 | m |  |  |  |
|  | N 130 | Ladder No. 3 | m |  |  |  |
|  | N 130 | Ladder No. 4 | m |  |  |  |
|  | N 130 | Ladder No. 5 | m |  |  |  |
|  | N 130 | Ladder No. 6 | m |  |  |  |
|  |  | # Stainless Steel Ladder |  |  |  |  |
|  | N 130 | Stainless Steel ladder No. 7 fabricated with 75 x 12 mm Stainless Steel stringers 375 mm apart and 20 mm dia Stainless Steel non-slip rungs at 300 mm centers, fixed complete to concrete as shown in detail on drawing. | m |  |  |  |
|  |  | Manhole Cover |  |  |  |  |
|  | N 29 | Locally fabricated stainless steel manhole cover 6 mm thick with 2 Nr. 40 x 6 mm stainless steel welded on top to serve as part of hinges and 75 x 75 x 10 mm stainless steel angle frame fixed on to 900 x 900 mm manhole opening provided on the concrete cover slab. The frame shall be fixed to concrete with 12 mm dia. rag bolts and nuts at suitable intervals. Locking arrangement shall also be provided complete as shown in detail on drawing | nr |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS N: MISCELLANEOUS METALWORK (CONDT.) |  |  |  |  |
|  | N 9 | 50 x 50 x 6 mm Stainless Steel angle bracket for S/S ladder fixed to concrete wall complete as shown in detail on drawing. | nr |  |  |  |
|  | N 9 | 50 x 6.5 mm Stainless Steel pipe bracket bent to shape of 150 mm dia. DI pipe and fixed to concrete with 12 mm dia. stainless steel rag bolt and nut as shown in detail on drawing. | nr |  |  |  |
|  | N 9 | 50 x 6.5 mm Stainless Steel pipe bracket bent to shape of 200 mm dia. DI pipe and fixed to concrete with 12 mm dia. stainless steel rag bolt and nut as shown in detail on drawing. | nr |  |  |  |
|  | N 9 | Fabricated and fixing of at stainless steel vent pipe as shown in detail Drawing No. W1072/TW/11 | nr |  |  |  |
|  |  | CLASS U: BRICK WORK BLOCK WORK AND MASONRY |  |  |  |  |
|  | U 121 | 225 mm thick brick masonry in cement mortar 1:5 in substructure up to DPC level. | m2 |  |  |  |
|  | U 121 | 225 mm thick brick masonry in cement mortar 1:5 in superstructure from DPC level to bracing No. 1 | m2 |  |  |  |
|  | U 121 | 225 mm thick brick masonry in cement mortar 1:5 in superstructure from bracing No. 1 to bracing No. 2 | m2 |  |  |  |
|  | U 121 | 225 mm thick brick masonry in cement mortar 1:5 in superstructure from bracing No. 2 to bracing No. 3 | m2 |  |  |  |
|  | U 121 | 225 mm thick brick masonry in cement mortar 1:5 in superstructure from bracing No. 3 to bottom of ring beam. | m2 |  |  |  |
|  |  | CLASS V – PAINTING |  |  |  |  |
|  | V 539 | Apply two coats of approved quality weather proof emulsion paint on external faces of walls, columns and beams including concrete louvers and reveals. | m2 |  |  |  |
|  | V 534 | Apply two coats of approved quality weather proof emulsion paint on soffits of beams, walls and slabs. | m2 |  |  |  |
|  | V 539 | Apply two coats of approved quality emulsion paint muffin colour on faces of internal walls, beams and columns including concrete louvers and reveals. | m2 |  |  |  |
|  | V 534 | Apply two coats of approved quality emulsion paint muffin colour on soffits of beams, walls and slabs. | m2 |  |  |  |
|  | V 534 | Apply two coats of approved quality reflective paint on top faces of roof slabs. | m2 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | # CLASS W: WATER PROOFING |  |  |  |  |
|  | W 166 | 20 mm thick Damp proof course in cement and sand mortar 1:2 on 225 mm wide brick masonry foundation. Rate to include for treating with two coats of hot tar blinded with sand. | m |  |  |  |
|  |  | CLASS X – MISCELLANEOUS WORK Supplying and fixing in position following items as directed |  |  |  |  |
|  | X 9 | Water level indicator comprising of 300 mm dia. 25 mm thick copper float with 3 Nr. 6 mm dia. copper hooks welded at equilaterally triangular positions, moving up and down on 3 Nr. Nylon guide wires of diameter less than 6 mm passing through eyes in copper float and anchored at top and bottom to 6 Nr. 12 mm dia. alloy steel eye bolts. (3 Nr. in each position) sunk in concrete. Copper float to be connected to the Aluminium weight with pointer painted in red and moving between framed timber guides of wooden gauge by Nylon cord rope passing over 4 Nr. 100 mm dia. wire shoot brass pulleys, fixed on concrete roof slab and walls. 40 mm thick Jack timber indicator board to be 150 mm wide painted in white and graduated in meters from 0 - 5.0 m and to be fixed on concrete at 8 points by No. 12 Brass screws and rawl plug all complete as shown in detail on drawing. The whole system shall be assembled and fixed in position to smooth working order. |  |  |  |  |
|  | X 9 | Sky light with 225 mm dia. and 12 mm thick armored glass and 6 mm thick mastic filling etc. on lantern roof. | nr |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CLASS Z – SIMPLE BUILDING WORKS INCIDENTAL TO CIVIL ENGINEERING WORKS |  |  |  |  |
|  |  | ## Carpentry and Joinery Structural and carcassing timber category ‘B’, All timber works shall be treated with two coats of approved quality wood preservative before fixing in position and to be painted with one coat of wood primer and two coats of enamel paint of approved quality and colour. |  |  |  |  |
|  |  | ## Windows, doors and glazing |  |  |  |  |
|  | Z 311 | 450 x 900 mm door (ventilator) on lantern wall consisting of 50 x 25 mm sash in 75 x 50 mm frame sashes shall be divided in to 6 Nr. Rectangular frames and fixed with stainless steel mesh as detailed on drawing. | nr |  |  |  |
|  | Z 311 | 450 x 600 mm window (ventilator) on lantern wall consisting of 50 x 25 mm sash in 75 x 50 mm frame sashes shall be divided in to 6 Nr. Rectangular frames and fixed with stainless steel mesh as detailed on drawing. | nr |  |  |  |
|  | Z 323 | 1125 x 2025 mm glazed panelled oxidized Aluminium door and frame complete as shown in detail on drawing. | nr |  |  |  |
|  |  | ## Surface finishes |  |  |  |  |
|  | Z 413 | 16 mm thick external and internal plastering on brick walls in cement lime and sand mortar 1:1:5 finished semi rough as specified including door and window reveals. | m2 |  |  |  |
|  | Z 411 | 12 mm thick cement rendering on concrete floor of tower in cement mortar 1:3 finished smooth with colored cement float. | m2 |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SIMPLE BUILDING WORKS INCIDENTAL TO CIVIL ENGINEERING WORKS (CONDT.) |  |  |  |  |
|  |  | ## Cabled building services |  |  |  |  |
|  |  | # ELECTRICAL INSTALLATION Note: All conduit pipes/ cables are to be concealed in walls and ceiling. Wiring to be completed upto M.C board. Contractor to carry necessary testing. All materials supplied, installed or fixed complete as directed. Rate to include for all PVC wiring involved as specified complete to working order. All fixtures shall be of approved quality and make to the satisfaction of the engineer and as specified in the specification |  |  |  |  |
|  | Z 781 | Single phase 30 amp main circuit breaker | Nr |  |  |  |
|  | Z 781 | Single phase 30 amp RCCB. | Nr |  |  |  |
|  | Z 781 | 15 amp miniature circuit breakers | Nr |  |  |  |
|  | Z 781 | 13 amp miniature circuit breakers | Nr |  |  |  |
|  | Z 781 | 12 way distribution board with plastic box having transparent front lid. | Nr |  |  |  |
|  | Z 772 | Final circuits for lighting points. | Nr |  |  |  |
|  | Z 772 | Final circuits for socket outlets. | Nr |  |  |  |
|  | Z 782 | 15 amp single One way switch. | Nr |  |  |  |
|  | Z 783 | Weather proof Fluorescent single lamp fitting (40 w) | Nr |  |  |  |
|  | Z 783 | Approved quality watertight halogen lamp. | Nr |  |  |  |
|  | Z 783 | Approved quality watertight bulk head lamp. | Nr |  |  |  |
|  | Z 783 | Approved quality watertight warning light. | Nr |  |  |  |
|  | Z 784 | 13 amp switched socket outlet flushed type | Nr |  |  |  |
|  | Z 762 | Earth electrode complete with 40 mm dia. 2.00 m long GI pipe driven in to ground for connecting frame work continuity conductor in order to obtain satisfactory earth resistance reading as required by the CEB. | Nr |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |

| Item No | CESMM Cord | Description | Unit | Qty. | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SIMPLE BUILDING WORKS INCIDENTAL TO CIVIL ENGINEERING WORKS (CONDT.) # ELECTRICAL INSTALLATION |  |  |  |  |
|  | Z 769 | Allow for providing and fixing lightning conductor complete with necessary copper tapes electrodes etc., complete as shown in detail on drawing and as directed. |  |  |  |  |
|  |  | Total of Page No. carried to Summery in Page No. |  |  |  |  |